GRADE 7 ROBOTICS & STEAM

Complete Curriculum Guide

Theme: "Sensors, Automation & Smart Systems"

National Academy of STEAM Education

Nepal

Academic Year 2083-84 B.S.

30 Students | 45 Minutes Weekly | 12 Automation Projects

Arduino Programming • Sensors • Motors • Real-World Automation

# Table of Contents

• Curriculum Overview

• Teaching Strategy

• Arduino Basics Guide

• Annual Calendar

• 12 Project Descriptions (ALL DETAILED)

• Budget & Costs

• Assessment

• Parent Communication

# 1. Curriculum Overview

Grade 7 students (ages 12-13) enter Arduino-based automation and sensor integration. Building on Grade 6, students now program microcontrollers to create smart systems responding to their environment.

### Learning Outcomes

• Write and upload Arduino code independently

• Integrate sensors to detect environmental conditions

• Control motors and actuators based on sensor input

• Design automated systems solving Nepal problems

• Debug code and troubleshoot hardware

# 2. Teaching Strategy (45 Minutes)

Arduino projects need programming + hardware. Solution: structured lab time with pre-prepared components and code templates. Projects span 2-3 weeks.

### Session Structure

• 0-5 min: Settling, review

• 5-12 min: Concept + code walkthrough

• 12-15 min: Teacher demo

• 15-40 min: Build, code, test (25 min)

• 40-45 min: Troubleshoot, homework

# 3. Arduino Basics

Week 1 covers: Arduino IDE, board anatomy, first Blink program, basic C++ syntax.

### Essential Concepts

• Setup & Loop: void setup() runs once, void loop() repeats

• Digital: HIGH/LOW for LEDs, buttons

• Analog: 0-1023 values for sensors

• Serial Monitor: Debugging tool

• If Statements: if (value > threshold) { action }

# 4. Annual Calendar (2083-84)

| Weeks | Project |
| --- | --- |
| 1 | Arduino Orientation |
| 2-4 | P1: Alcohol Detector |
| 5-7 | P2: Traffic Light |
| 8-10 | P3: Auto Irrigation |
| 11-13 | P4: Hand Dryer |
| 14-16 | P5: Conveyor Belt |
| 17-18 | DASHAIN |
| 19-21 | P6: Motor Dhiki |
| 22-24 | P7: Smart Dustbin |
| 25-27 | P8: Motor Speed Control |
| 28-29 | P9: 3D Rocket Model |
| 30-31 | P10: Rocket Launch Sim |
| 32-33 | P11: System Integration |
| 34-36 | P12: Final + SHOWCASE |

# 5. The 12 Projects - Complete Details

TERM 1: SENSOR-BASED SMART SYSTEMS

## Project 1: Alcohol Detector System

Duration: 3 weeks | Build Time: 80-90 min

Complexity: 5/10

Learning Focus: MQ-3 alcohol sensor, analog input, threshold programming, safety systems

Description: Build alcohol detection system using MQ-3 sensor. When alcohol detected above threshold, LED turns red and buzzer sounds. Learn analog sensor reading (0-1023), thresholds, if-then logic. Real application: breathalyzer concept.

Materials: Arduino Uno, MQ-3 alcohol sensor module, red LED, buzzer, 220Ω resistor, breadboard, jumper wires, USB cable, cardboard housing, hand sanitizer (testing)

Weekly Breakdown:

Week 1: Intro to MQ-3 sensor and analog input, wire sensor to A0, upload test code to read values via Serial Monitor, understand 0-1023 range

Week 2: Add LED and buzzer outputs, write if-then code (if sensorValue > 400 trigger alarm), test with alcohol vapor, calibrate threshold

Week 3: Create housing at home, mount sensor and outputs, test reliability, present with drunk driving prevention discussion

Arduino Code: analogRead(A0), if (value > threshold), digitalWrite, Serial.println for debugging

Nepal Context: Traffic safety in Nepal; drunk driving prevention; breathalyzer technology; vehicle safety; law enforcement applications; public health awareness

## Project 2: Automatic Traffic Light

Duration: 3 weeks | Build Time: 70-90 min

Complexity: 7/10

Learning Focus: Sequential timing, delay functions, LED control, traffic system logic

Description: Create fully automated traffic light cycling Red → Yellow → Green → Yellow → Red. Learn timing control with delay(), sequential programming, realistic traffic patterns. Program realistic timing (Red 30s, Green 20s, Yellow 5s).

Materials: Arduino Uno, 3 LEDs (red, yellow, green), 3× 220Ω resistors, breadboard, jumper wires, cardboard traffic light housing, optional: pedestrian button

Weekly Breakdown:

Week 1: Wire 3 LEDs to digital pins, learn digitalWrite and delay, program basic sequence (R:5s, Y:2s, G:5s, repeat)

Week 2: Adjust to realistic timing (R:30s, Y:5s, G:20s, Y:5s), add comments explaining each phase, test cycle with stopwatch

Week 3: Build traffic light housing at home, mount LEDs vertically, optional: add pedestrian button extending Green, present with Nepal traffic discussion

Arduino Code: digitalWrite for LED control, delay for timing, sequence logic, optional: digitalRead for button

Nepal Context: Kathmandu traffic management; intersection safety; reducing manual traffic police burden; smart city concepts; synchronized signals; pedestrian safety; reducing accidents

## Project 3: Water Sprinkle System (Auto Irrigation)

Duration: 3 weeks | Build Time: 90 min

Complexity: 8/10

Learning Focus: Soil moisture sensor, relay module, pump control, agricultural automation

Description: Build automated irrigation system using soil moisture sensor. When soil is dry (below threshold), Arduino activates relay to turn ON water pump. When moist, pump turns OFF. Real farming application, water conservation.

Materials: Arduino Uno, soil moisture sensor module, 5V relay module, small water pump (3-6V), power supply/battery, tubing, water container, plant pot with soil, jumper wires

Weekly Breakdown:

Week 1: Learn soil moisture sensor, test in dry vs wet soil, read analog values (0-1023), understand moisture threshold

Week 2: Wire relay to Arduino, connect pump to relay, write code: if (moisture < 300) pump ON else OFF, test system

Week 3: Create complete irrigation setup at home with plant pot, calibrate threshold for actual soil, add LED indicator, present water-saving benefits

Arduino Code: analogRead for moisture sensor, relay control with digitalWrite, threshold comparison, Serial.println for monitoring

Nepal Context: Agriculture automation for Nepali farmers; water conservation in dry seasons; smart farming; reducing manual labor; increasing crop yield; precision agriculture

## Project 4: Automatic Hand Dryer

Duration: 3 weeks | Build Time: 80-90 min

Complexity: 6/10

Learning Focus: Ultrasonic sensor (HC-SR04), distance measurement, motor/fan control, hygiene automation

Description: Build automatic hand dryer using ultrasonic sensor to detect hands. When hands placed under sensor (distance < 10cm), fan turns ON. When removed, fan turns OFF. Learn distance measurement with ultrasonic.

Materials: Arduino Uno, HC-SR04 ultrasonic sensor, small DC fan (5V), transistor or relay for fan control, breadboard, jumper wires, cardboard housing

Weekly Breakdown:

Week 1: Intro to ultrasonic sensor (measures distance with sound waves), wire sensor (Trig, Echo pins), test distance measurement with Serial Monitor

Week 2: Add fan motor, write code: if (distance < 10) fan ON else OFF, test hand detection, adjust distance threshold

Week 3: Build hand dryer housing at home (sensor on top, fan below), mount all components, test reliability, discuss public hygiene applications

Arduino Code: Ultrasonic distance calculation (pulseIn, duration * 0.034 / 2), conditional motor control, distance threshold

Nepal Context: Public toilet hygiene; touchless technology in hospitals; COVID-19 contactless solutions; reducing disease transmission; modern sanitation in Nepal

TERM 2: MECHANICAL + MOTOR SYSTEMS

## Project 5: Conveyor Belt System

Duration: 3 weeks | Build Time: 90 min

Complexity: 7/10

Learning Focus: DC motor control, belt mechanism, industrial automation, ON/OFF control

Description: Build mini conveyor belt using DC motor. Belt moves objects from one end to other. Add ON/OFF button control. Learn industrial automation, manufacturing, assembly lines.

Materials: Arduino Uno, DC motor, motor driver (L293D), rubber belt or fabric strip, cardboard platform, 2 rollers (pencils/dowels), push button, jumper wires, 9V battery

Weekly Breakdown:

Week 1: Build conveyor frame with rollers and belt, connect motor to driver, test motor spinning belt, learn motor driver connections

Week 2: Add button control (press to start, press again to stop), write toggle code, test conveyor with small objects (coins, erasers)

Week 3: Improve belt tension and speed at home, add LED indicator (green when running), present with factories and automation discussion

Arduino Code: Motor driver control (digitalWrite to IN pins), button toggle logic with state variable, debouncing

Nepal Context: Garment factory automation in Nepal; tea processing plants; manufacturing efficiency; reducing manual labor; industrial growth; assembly line concepts

## Project 6: Motor-Powered Dhiki (Traditional Tool Automation)

Duration: 3 weeks | Build Time: 80-90 min

Complexity: 6/10

Learning Focus: Cultural technology integration, lever mechanism, motor automation, traditional tool modernization

Description: Automate traditional Nepali Dhiki (foot-powered rice mill) using DC motor. Motor replaces foot power to operate lever mechanism. STRONG Nepal cultural relevance - preserving tradition with technology.

Materials: Arduino Uno, DC motor, motor driver, cardboard/wood for dhiki structure, lever mechanism (popsicle sticks), pivot point, small container (grain/rice simulation), switch

Weekly Breakdown:

Week 1: Learn about traditional Dhiki and how it works, build manual lever mechanism (foot-powered), understand up-down motion

Week 2: Replace foot power with motor, connect motor to lever via cam or crank mechanism, program motor for repeated up-down motion

Week 3: Refine mechanism at home, test with actual rice/grain, decorate with traditional Nepali designs, present cultural + tech integration

Arduino Code: Motor continuous rotation control, optional: speed adjustment with PWM, ON/OFF button

Nepal Context: STRONG NEPAL RELEVANCE - Traditional dhiki in villages; modernizing cultural tools; preserving heritage with technology; rural women's labor reduction; cultural pride + innovation

## Project 7: Smart Dustbin (Auto-Open Lid)

Duration: 3 weeks | Build Time: 90 min

Complexity: 6/10

Learning Focus: Servo motor control, ultrasonic sensor, automated lid mechanism, waste management innovation

Description: Build smart dustbin with automatic lid. When hand approaches (ultrasonic detects < 15cm), servo motor opens lid. After few seconds, lid closes automatically. Touchless waste disposal.

Materials: Arduino Uno, HC-SR04 ultrasonic sensor, SG90 servo motor, cardboard dustbin, servo mount bracket, jumper wires, USB cable

Weekly Breakdown:

Week 1: Learn servo motor basics (0-180 degrees), test servo movement with servo.write(), wire ultrasonic sensor, test distance detection

Week 2: Write code: when hand detected, servo opens lid (90°), wait 3 seconds, close lid (0°), test multiple cycles

Week 3: Build complete dustbin housing at home, mount sensor on top, attach servo to lid, test reliability, discuss waste management

Arduino Code: #include <Servo.h>, servo.attach(), servo.write(), ultrasonic distance, delay for auto-close timing

Nepal Context: Waste management in Kathmandu; touchless technology for hygiene; reducing disease transmission from trash; smart city waste solutions; public bin innovation

## Project 8: Motor Speed Control System (PWM)

Duration: 3 weeks | Build Time: 80-90 min

Complexity: 5/10

Learning Focus: PWM (Pulse Width Modulation), variable speed control, potentiometer input, analog control

Description: Control DC motor speed using potentiometer (dial/knob). Turn knob left = slow, right = fast. Learn PWM concept - how Arduino simulates variable voltage (0-255) to control motor speed. IMPORTANT concept for future projects.

Materials: Arduino Uno, DC motor, motor driver or transistor, potentiometer (10kΩ), breadboard, jumper wires, 9V battery

Weekly Breakdown:

Week 1: Learn PWM concept (analogWrite 0-255), test LED brightness control with potentiometer, understand analog input mapping

Week 2: Connect motor driver, map potentiometer value (0-1023) to PWM (0-255), control motor speed with knob, observe speed changes

Week 3: Build control panel at home with mounted potentiometer, add LED speed indicator (brighter = faster), present PWM applications

Arduino Code: analogRead for potentiometer, map(value, 0, 1023, 0, 255), analogWrite for PWM motor control

Nepal Context: Fan speed control in homes; water pump regulation; industrial machine speed adjustment; energy efficiency; variable speed applications in daily life

TERM 3: CREATIVE + INTEGRATED SYSTEMS

## Project 9: 3D Rocket Model (STEAM Design Focus)

Duration: 2 weeks | Build Time: 60-75 min

Complexity: 6/10

Learning Focus: Design thinking, 3D structure building, creativity + engineering, aesthetics, no electronics

Description: Design and build 3D rocket model from cardboard. Focus on creativity, design, structure. Learn rocket parts (nose cone, body, fins, engine). Prepares for Project 10 where electronics added. STEAM integration: Science (rockets), Tech (next), Engineering (structure), Art (design), Math (measurements).

Materials: Cardboard, scissors, glue, ruler, compass, markers/paint, decorative materials, reference images of rockets

Weekly Breakdown:

Week 1: Learn rocket basics and parts, sketch rocket design, calculate dimensions (height, diameter, fin angles), create templates

Week 2: Build 3D rocket at home using templates, assemble all parts (nose cone, body tube, fins, base), decorate with Nepal flag or colors, present rocket designs

Arduino Code: None (pure design project)

Nepal Context: Nepal's space ambitions (NepaliSat-1 satellite); STEM education; inspiring future aerospace engineers; national pride in space technology; design thinking

## Project 10: Rocket Launch Simulation (LED + Sound)

Duration: 2 weeks | Build Time: 80-90 min

Complexity: 6/10

Learning Focus: Countdown sequence, LED patterns, buzzer tones, event simulation, sequential programming

Description: Add electronics to rocket from Project 9. Create launch simulation: button press starts 10-second countdown (display with Serial Monitor), buzzer beeps each second, at zero = rapid LED flashing + continuous buzzer (liftoff simulation). Exciting project combining creativity + code.

Materials: Arduino Uno, 3-5 LEDs (white/red/orange), buzzer, push button, resistors, breadboard, jumper wires (mount inside/beside rocket model)

Weekly Breakdown:

Week 1: Wire LEDs and buzzer, program countdown (10 to 0) with Serial Monitor display, buzzer beep each second, LED blinks

Week 2: Integrate electronics with rocket model from P9, add button to start countdown, create liftoff effect (all LEDs flash rapidly, buzzer loud tone), present launch simulation

Arduino Code: for loop for countdown, Serial.print for display, tone() function for buzzer notes, LED patterns, button trigger

Nepal Context: Inspiring interest in space technology; NepaliSat-1 (Nepal's first satellite 2019); STEM career inspiration; countdown events (New Year, festivals); celebration technology

## Project 11: Smart System Integration (Mini Project)

Duration: 2 weeks | Build Time: 90 min

Complexity: 5/10

Learning Focus: System integration, combining previous projects, multi-sensor systems, creative problem-solving

Description: Combine 2 previous systems into one smart project. Examples: Smart Dustbin + Alert (ultrasonic opens lid + buzzer alerts when full), Auto Hand Dryer + Counter (counts washes), Irrigation + Alert (waters plant + LED shows moisture). Students choose which to integrate.

Materials: Components from 2 previous projects (students choose), additional LEDs/sensors as needed, breadboard, wires

Weekly Breakdown:

Week 1: Plan integration, choose 2 systems to combine, draw system diagram showing inputs/outputs, write pseudocode for combined logic

Week 2: Build integrated system, test both functions working together, debug issues, present combined smart system with explanation of how integration adds value

Arduino Code: Multiple sensor inputs, combined if-then logic, managing multiple outputs, state machines if needed

Nepal Context: Smart systems in Nepal; IoT (Internet of Things) concepts; building integrated solutions for Nepal problems; multi-function devices; efficient technology use

## Project 12: FINAL PROJECT - Real-Life Automation Model

Duration: 3 weeks | Build Time: 90+ min

Complexity: 7/10

Learning Focus: Applying all year's learning, system design, presentation skills, real-world problem-solving

Description: Students design automation solving real Nepal problem. Choose ONE: (1) Smart Home - auto lights, security, climate; (2) Smart Farm - irrigation, monitoring, automation; (3) Smart City - traffic, waste, energy. Must include: 2+ sensors, 1+ actuator, decision-making code, housing/model, presentation explaining problem + solution.

Materials: All components from previous projects available for reuse, additional sensors/motors as needed, cardboard for models, presentation materials

Weekly Breakdown:

Week 1: Identify problem in chosen category, research Nepal context, sketch system design, plan components needed, write code outline

Week 2: Build system, integrate all components, test functionality, debug and improve, create model housing/environment

Week 3: Finalize project at home, prepare presentation (what problem, how it solves, Nepal impact), rehearse explanation, SHOWCASE EVENT - present to parents/teachers/school, demonstrate working system, explain automation value

Arduino Code: All year's concepts: sensors, motors, servos, relays, if-then logic, PWM, multi-system integration

Nepal Context: Solving real Nepal challenges: smart homes for energy efficiency; agricultural automation for food security; smart cities for urban management; demonstrating student innovation; inspiring community; showcasing STEAM education value

# 6. Material Costs & Budget (Detailed)

Costs for 30 students (6 groups). Includes ~15-20% markup for import duties, replacements, storage. Based on 2083 B.S. Kathmandu market rates.

| Component/Project | Per Group | Total (6) |
| --- | --- | --- |
| CORE KIT (ONE-TIME) |  |  |
| Arduino Uno R3 + USB | Rs. 1,200 | Rs. 7,200 |
| Breadboard 830 | Rs. 180 | Rs. 1,080 |
| Jumper Wire Set | Rs. 150 | Rs. 900 |
| PROJECT COMPONENTS |  |  |
| P1: MQ-3 + LED + Buzzer | Rs. 350 | Rs. 2,100 |
| P2: 3 LEDs + Resistors | Rs. 80 | Rs. 480 |
| P3: Moisture + Relay + Pump | Rs. 550 | Rs. 3,300 |
| P4: Ultrasonic + Fan | Rs. 320 | Rs. 1,920 |
| P5: Motor + Belt | Rs. 280 | Rs. 1,680 |
| P6: Motor + Dhiki Parts | Rs. 320 | Rs. 1,920 |
| P7: Servo + Ultrasonic | Rs. 420 | Rs. 2,520 |
| P8: Pot + Motor Driver | Rs. 180 | Rs. 1,080 |
| P9-10: LEDs, Buzzer, Card | Rs. 150 | Rs. 900 |
| P11-12: Integration | Rs. 200 | Rs. 1,200 |
| Resistor Pack + Consumables | Rs. 250 | Rs. 1,500 |
| TOTAL BUDGET | Rs. 4,680 | Rs. 28,080 |

Budget Summary:

Total: Rs. 28,080 for 30 students

Per Student: Rs. 936 for entire year

Note: Arduino kits reusable across projects. Components can be salvaged for Grade 8.

# 7. Assessment

No exams. Assessment: Functionality 30%, Code Quality 25%, Understanding 20%, Innovation 15%, Collaboration 10%.

_______________

Grade 7: From Learners to Makers

Students master Arduino, sensors, and automation,

creating smart systems solving real Nepal challenges.

Building Nepal's automation future!

National Academy of STEAM Education • Kathmandu, Nepal